How to crack WPA/WPA2

Introduction

- There is another important difference between cracking WPA/WPA2 and WEP. This is the approach used to crack the WPA/WPA2 pre-shared key. Unlike WEP, where statistical methods can be used to speed up the cracking process, only plain brute force techniques can be used against WPA/WPA2.

- That is, because the key is not static, so collecting IVs like when cracking WEP encryption, does not speed up the attack. Wi-Fi Protected Access(WPA) was created to solve the gaping security flaws that plagued WEP. Perhaps the most predominant flaw in WEP is that the key is not hashed, but concatenated to the IV, allowing completely passive compromise of the network.

- WPA solves this problem by rotating the key on a per-packet basis, which renders the above method useless. However, nothing is perfectly secure, and WPA-PSK is particularly vulnerable during client association, during which the hashed network key is exchanged and validated in a "four-way handshake".

Step 1:

| Objective | To identify wireless interface |
| --- | --- |
| Command | # iwconfig |

1

2

3

Descriptionn :

- Lo is the local host interface.

- Eth0 is the Ethernet interface.

- Wlan0 is the wireless interface.

Step 2 :

| Objective | To display a list of detected access points and a list of connected clients (“station”). |
| --- | --- |
| Command | Airodump-ng wlan0 |

The system will respond :

| No | Field | Description |
| --- | --- | --- |
| 1. | BSSID | MAC address of the access point. In the Client section, a BSSID of ”(not associated)” means that the client is not associated with any AP. In this unassociated state, it is searching for an AP to connect with. |
| 2. | PWR | Signal level reported by the card. Its signification depends on the driver, but as the signal gets higher you get closer to the AP or the station. |
| 3. | Beacons | Number of announcements packets sent by the AP. Each access point sends about ten beacons per second at the lowest rate (1M), so they can usually be picked up from very far. |
| 4. | # Data | Number of captured data packets (if WEP, unique IV count), including data broadcast packets. |
| 5. | #/s | Number of data packets per second measure over the last 10 seconds. |
| 6. | CH | Channel number (taken from beacon packets). Note: sometimes packets from other channels are captured even if airodump-ng is not hopping, because of radio interference. |
| 7. | MB | Maximum speed supported by the AP. If MB = 11, it's 802.11b, if MB = 22 it's 802.11b+ and higher rates are 802.11g. The dot (after 54 above) indicates short preamble is supported. |
| 8. | ENC | Encryption algorithm in use. OPN = no encryption,”WEP?” = WEP or higher (not enough data to choose between WEP and WPA/WPA2), WEP (without the question mark) indicates static or dynamic WEP, and WPA or WPA2 if TKIP or CCMP is present. |
| 9. | CIPHER | The cipher detected. One of CCMP, WRAP, TKIP, WEP, WEP40, or WEP104. Not mandatory, but TKIP is typically used with WPA and CCMP is typically used with WPA2. |
| 10. | AUTH | The authentication protocol used. One of MGT (WPA/WPA2 using a separate authentication server), SKA (shared key for WEP), PSK (pre-shared key for WPA/WPA2)and etc. |
| 11. | ESSID | The so-called “SSID”, which can be empty if SSID hiding is activated. In this case, airodump-ng will try to recover the SSID from probe responses and association requests. |
| 12. | STATION | MAC address of each associated station or stations searching for an AP to connect with. |
| 13. | Lost | The number of data packets lost over the last 10 seconds based on the sequence number. |
| 14. | Packets | The number of data packets sent by the client. |
| 15. | Probes | The ESSIDs probed by the client. These are the networks the client is trying to connect to if it is not currently connected. |

Step 3 :

| Objective | Start to capturing traffic for the target access point and prepare to deauthenticate client. |
| --- | --- |
| Command | Airodump-ng -c11 --bssid 00:18:39:6A:BE:D4 -w netsec wlan0 |

The system will respond:

Step 4:

| Objective | To deauthenticate the client who is already connected and force them to re-exchange the WPA key. |
| --- | --- |
| Command | Aireplay-ng -0 5 -a 00:18:39:6A:BE:D4 -c 00:16:B6:5B:90:E8 wlan0 |

- Client acknowledges our deauthentication packets connects back to wireless access point to exchange 4-way handshake  of WPA key that we will capture.

- -c 00:16:B6:5B:90:E8 is client MAC connected to wireless access point.

- –a =XXXXXXXXXXXXXXXXXXXXXXXXX

- -0 5 =

- Wlan0 =

1

- Airodump will show the captured handshake in the top right hand corner. Now that we captured the packet you need, we can close this window and proceed to break the WPA key or just let it open and open a new windows.

Step 5 :

| Objective | To prepare dictionary for the attack |
| --- | --- |
| command | Vim netsec.txt |

Sample of dictionary:

- For proof of concept, we use dictionary based words or phrases we should compile a dictionary file and include the real password somewhere in the middle. For the attack we can download a good dictionary from freebsd.

Step 6 :

| Objective | Run aircrack-ng and break key with the dictionary we compiled and the 4-way handshake we captured. |
| --- | --- |
| command | Aircrack-ng -w netsec.txt *.cap |

2

1

3

Explanation :

- -w netsec.txt is the name of dictionary file.

- *.cap is name of group of file containing the captured packets. Notice in this case that we used the wildcard * to include multiple files.

- The only thing that does give the information to start an attack is the handshake between client and AP. Handshaking is done when the client connects to the network.

Finally, here is what successfully cracking the pre-shared key look like :

password

- WPA-TKIP key has been successfully decrypted. Again, all it takes is capturing a single handshake and then running a dictionary attack in our spare time.